# LYCAN LN635L

# Thermal Scope

Thermal Image Scope is equipped with a 384 x 288 thermal detector and a 1024 × 768 AMOLED display. Boasting ultra-high thermal sensitivity, it delivers clear target recognition even in harsh weather and low temperature difference scenarios. Featuring long battery life and a maximum observation distance of 2600m, this scope is ideal for hunting, outdoor patrol, search and rescue.

Key Features

- 384×288 resolution with 12 μm sensor, 50Hz frame rate

- High sensitivity thermal module NETD ≤ 15 mK (25°C, F# = 0.9)

- 1024 × 768 resolution 0.49-inch AMOLED display

- 35mm F1.0 lens, observation distance up to 2600 m

- Up to 10 hours of continuous operation at 25°C

- 4 replaceable and rechargeable Li-ion batteries, battery non-polar design for normal operation in forward and reverse installation

- Built-in 905nm Class 1 laser rangefinder (1.2km, ±1m accuracy) and ballistic calculation

- Supports audio recording, snapshot and recoil-activated recording

- IP67 protection grade, operating temperature: -20°C ~ +50°C

- High shock resistance up to 1000 g/0.4 ms

- Wi-Fi connection and 64GB built-in storage available

n Specification

| Item category | LYCAN LN335L |
| --- | --- |
| Thermal Module |  |
| Image Sensor | UFPA (VOx) |
| Resolution | 384×288 |
| Pixel Pitch | 12µm |
| Frame Rate | 50Hz |
| NETD | ≤18mk |
| Spectral Range | 8-14µm |
| Optical Parameter |  |
| Focal length / Aperture | 35mm; F1.0 |
| Field of View | 7.5°×5.6° |
| Base Magnification | 3.5X |
| Focusing Mode | Focus Knob |
| Eye Relief | 48mm |
| Exit Pupil | 6mm |
| Diopter | +5D to -5D |
| Detection Range | 1800m (Human; 3pixels) |
| Recognition Range | 450m (Human; 12pixels) |
| Image Display |  |
| Screen | AMOLED, 0.49 inch |
| Resolution | 1920×1080 |
| Screen Brightness Adjustment | Support |
| Brightness Adjustment | Support |
| Contrast Adjustment | Support |
| Image Enhancement | Support |
| Scene Mode | City;Plain;Forest |
| Display Mode | Circle;Box |
| Color Palette | White Hot/Black Hot/Red Hot/Fusion/Golden Hot etc. |
| Digital Zoom | 1×; 2×; 4×; 8× |
| Power Supply |  |
| Battery type & Number | 4 replaceable 18350 Lithium batteries |
| External Power Supply | DC 5V (USB Type-C) |
| Power Consumption | ≤1.9W（25℃） |
| Battery Operating Time | ≥12h （25℃） |
| System |  |
| Record Video | Support |
| Capture Snapshot | Support |
| Audio Recording | Support |
| Local Album | Support |
| Recoil Activation Recording | Support |
| Picture in Picture | Support |
| Hot Track | Support |
| Zeroing Adaptation Mode | Support |
| Screen Adaptation | - |
| Standby Mode | Support |
| Ballistic Calculation | Support |
| Ballistic Calibration | Support |
| Distance Measurement | Support |
| Zeroing Profiles | 10 |
| Reticle Pattern | 8 |
| Reticle Pattern | 6 |
| Aim Point Type | 3 |
| Aim Point Colour | 6 |
| BC Aim Point Type | 5 |
| BC Aim Point Colour | 4 |
| Ranging Cursor Type | 3 |
| Ranging Cursor Colour | 3 |
| Peripheral |  |
| Electronic Compass | Support |
| Attitude Sensor | Support |
| WIFI /APP | Support; 2.4G |
| Storage | Built-in EMMC (64GB) |
| Laser Rangefinder | 905nm;Class 1 Laser Safety |
| Ranging Distance & Accuracy | 1.2 Km*; ±1m |
| General |  |
| Data Transmission Interface | USB (Type-C) |
| Protection Level | IP67 |
| Operating Temperature | -20°C to +50°C |
| Max. Recoil | 1000g/0.4ms |
| Weight（without battery） | ≤595g |
| Dimension | 238×80×65（mm） |