# STAG SG650L

# Thermal Monocular

With a native magnification of 2.7x, a high-resolution 640 × 512 detector, and a premium 50 mm F0.9 lens, the C650L delivers exceptional performance for discerning hunters and long-range observation. Its 8.7°×7.0° field of view and a detection range of up to 2600 m ensure crisp, detailed imagery even in vast, open landscapes, making it the ultimate choice for users seeking professional-grade quality.

Key Features

● High-sensitivity thermal module with NETD ≤ 15 mK (@25° C, F# = 1.0).

● 50 mm, F0.9 lens with thermal detection range up to 2600 m.

● Advanced image enhancement algorithms with 1×, 2×, and 4× digital zoom.

● Built-in 905nm Class 1 laser rangefinder, providing accurate range data up to 1.2 km with ±1m precision.

● 0.39-inch AMOLED display with 1024 × 768 resolution for sharp, clear imagery.

● 1 replaceable 21700 lithium battery offering ≥7.5 hours of continuous operation at 25°C.

● Robust IP67 protection level, designed to withstand harsh field conditions.

n Specification

| Thermal Module |  |
| --- | --- |
| Image Sensor | UFPA (VOx) |
| Resolution | 640×512 |
| Pixel Pitch | 12µm |
| Frame Rate | 50Hz |
| NETD | ≤15mk |
| Spectral Range | 8-14µm |
| Optical Parameter |  |
| Focal length / Aperture | 50mm; F0.9 |
| Field of View | 8.7°×7.0° |
| Base Magnification | 2.7X |
| Focusing Mode | Focus Roller |
| Eye Relief | 15mm |
| Exit Pupil | 6mm |
| Diopter | +5D to -5D |
| Detection Range | 2600m (Human; 3pixels) |
| Recognition Range | 640m (Human; 12pixels) |
| Image Display |  |
| Screen | AMOLED, 0.39 inch |
| Resolution | 1024×768 |
| Screen Brightness Adjustment | Support |
| Brightness Adjustment | Support |
| Contrast Adjustment | Support |
| Image Enhancement | Support |
| Scene Mode | City;Plain;Forest |
| Display Mode | Circle;Box |
| Color Palette | White Hot/Black Hot/Red Hot/Fusion/Golden Hot etc. |
| Digital Zoom | 1×; 2×; 4× |
| Power Supply |  |
| Battery type & Number | 1 replaceable 21700 Lithium battery |
| External Power Supply | DC 5V (USB Type-C) |
| Battery Operating Time | ≥7.5h （25℃） |
| System |  |
| Record Video | Support |
| Capture Snapshot | Support |
| Audio Recording | Support |
| Local Album | Support |
| Recoil Activation Recording | - |
| Picture in Picture | Support |
| Hot Track | Support |
| Zeroing Adaptation Mode | - |
| Screen Adaptation | Support |
| Standby Mode | Support |
| Ballistic Calculation | - |
| Ballistic Calibration | - |
| Distance Measurement | Support |
| Zeroing Profiles | - |
| Reticle Pattern | - |
| Reticle Pattern | - |
| Aim Point Type | - |
| Aim Point Colour | - |
| BC Aim Point Type | - |
| BC Aim Point Colour | - |
| Ranging Cursor Type | 3 |
| Ranging Cursor Colour | 3 |
| Peripheral |  |
| Electronic Compass | Support |
| Attitude Sensor | Support |
| WIFI /APP | Support; 2.4G |
| Storage | Built-in EMMC (64GB) |
| Laser Rangefinder | 905nm;Class 1 Laser Safety |
| Ranging Distance & Accuracy | 1.2 Km*; ±1m |
| General |  |
| Data Transmission Interface | USB (Type-C) |
| Protection Level | IP67 |
| Operating Temperature | -20°C to +50°C |
| Max. Recoil | - |
| Weight（without battery） | ≤715g |
| Dimension | 218×82×70（mm） |